# Circuits and Control Coursework

Counter-clockwise rotations are positive

# Variables

d = natural length of the spring

x = Distance b/w the centre of the ball and the wall

δ = Distance b/w the centre of the electromagnet and the wall

y = distance b/w the centre of the ball and the centre of the electromagnet

k = elastic spring of stiffness

b = viscous damping coefficient

m = mass of the ball

g = acceleration due to gravity

r = radius of the ball

R = Resistance

V = Applied voltage

L0 = nominal inductance

L1 = positive constants

α = positive constants

c = positive constant

I = The current that runs through the circuit

φ = angle of incline

# Forces

## Force from spring

Fspring = kex

ex = x – d

Fspring = k(x - d)

Where:

- k = elastic spring of stiffness

- ex = extension

- x = Distance b/w the centre of the ball and the wall

- d = natural length of the spring

## Force from damper

Fdamper = bx

Where:

- b = Viscous damping coefficient

- x = Velocity of the ball

## Weight of ball

N = mgcosφ = 0

Fball =  mgsinφ

Where:

- N = Reaction

- m = mass

- g = acceleration due to gravity

- φ = angle of incline

- I = Moment of inertia

- r = radius of the ball

- T = Torque of the ball that causes rotation

## Torque of Ball (Force that causes rotation

I = 25mr2

FTorque = Iφr

FTorque = 25mr2 x φr

FTorque = 2mr2φ5r

FTorque = 2mrφ5

FTorque = 2mx5

Where:

- I = Moment of inertia

- m = mass

- r = radius of the ball

## Force from electromagnet

V = LI + IR

I = VL - IRL

I = 1L(V - IR)

I = V - ILR

L = L0 + L1 exp(−αy)

Fmag = c I2y2

y = δ - x

Where:

- L = Inductance

- L0 = nominal inductance

- L1 = Positive constants

- α = Positive constant

- y = Distance b/w the centre of the ball and the centre of the electromagnet

- c = Positive constant

- I = Current

- δ = Distance b/w the centre of the electromagnet and the wall

- x = Distance b/w the centre of the ball and the wall

## Total Force

Ftot = - Fspring - Fdamper - FTorque + FWeight + Fmag

mx = -k(x - d) - bx + 2mrφ5 + mgsinφ + c I2y2

x = rφ

mx2 = -k(x1 – d) - bx2 + 2mrφ5  + mgsinφ + c I2y2

mx2 = -k(x1 – d) - bx2 + 2mx5 + mgsinφ + c I2y2

mx2 - 2mx25 = -k(x1 – d) - bx2 + mgsinφ + c I2y2

(5m- 2m)x25 = -k(x1 – d) - bx2 + mgsinφ + c I2y2

3mx25 = -k(x1 – d) - bx2 + mgsinφ + c I2y2

x2 = 53m(-k(x1 – d) - bx2 + mgsinφ + cI2y2)

# State Space Representation

| x = [ x1x2I ] | x = [ x253m (-kx1 – d-bx2+ mgsinφ+ c I2(δ - x1)2)1LO + L1 e-αy(V - IR) ] |
| --- | --- |

# Characterise the Equilibrium Points of the System Using Proposition

The input of the system is the voltage across the circuit and the output is the position of the ball on the inclined plane.

x2e = 0

53m  (-kx1e – d-bx2e+ mgsinφ+ c (Ie)2δ - x1e2 = 0

1LO + L1 e-αyVe - IeR = 0

# Linearise the System at an Equilibrium Point (Use Deviation Variables)

|  | x = x2 |  |
| --- | --- | --- |
| - | 0 = x2e |  |
|  | x = x2 - x2e x = x2 | x2 = x2 - x2e |

|  | x = 53m (-kx1 – d-bx2+ mgsinφ+ c I2(δ - x1)2 ) |  |
| --- | --- | --- |
| - | 0 = 53m (-kx1e – d-bx2e+ mgsinφ+ c Ie2δ - x1e2 ) |  |
|  | x = 53m (-k(x1- x1e) - b(x2- x2e )) + c I2(δ - x1)2 - c Ie2δ - x1e2 ) x = 53m (-kx1 - bx2 + c I2(δ - x1)2 - c Ie2δ - x1e2) | x2 = x2 - x2e |

|  | I = 1LO + L1 e-αy(V - IR) |  |
| --- | --- | --- |
| - | 0 = 1LO + L1 e-αy(Ve - IeR) |  |
|  | I = 1LO + L1 e-αy (V - Ve - (I - Ie)R) I = 1LO + L1 e-αy (V - IR) | V = V - Ve |